Issue BZA

- Obtain a charged BZA, Hose, Cassette and New Filter paper.

  - Use tweezers to install new filter into the cassette.

  - Ensure the blue paper separator is not with the filter.

  - Ensure the black gasket is in the cassette holder top.

  - Do not over tighten the cassette holder.

- Attach cassette to BREATHING ZONE of wearer (shirt collar).

- Attach pump to belt.

- Press ON/OFF to start pump.

- Wait for the pump to complete a self test and to stabilize to the desired flow rate (typically 2.0 lpm).

- Note the Start Time and Flow Rate.

- To pause the pump run time Press PAUSE/HOLD.

- Press PAUSE/HOLD to resume run time.

- When departing from the area Press ON/OFF to stop the run time and pump.

- Turn pump into designated Field Team Member to process the filter.

Receiving and processing the filter paper

- A Field Team Member will be assigned responsibility to receive the BZA filters count on a scaler and complete a FRM-0108F “Radiological Survey Report – Breathing Zone Air (BZA) Spreadsheet” form (this form originates from the M&O contractor of the NNSS).

- Follow the instructions on the back of the form.

- Up to 6 people can be put on one FRM-0108F.  A new form will be created for each day.

- If the pump was issued to a team, record each team member’s name in the other columns.

- The Health & Safety division will maintain an electronic copy of each FRM-0108F.

- Record the required information on a draft paper version of FRM-0108F until you have access to a computer and the electronic FRM-0108F is available.  Follow the Instructions on the back of the form.

- As the team returns and turns in the BZA filter, record the information before the team is released.  Follow the instructions on the back of the form.

- Using tweezers, remove the filter paper and place into an envelope.

  - Label the envelope with the name and date of the filter.

- At this time, you can release the Team.

- Charge the BZA battery for the next use.

  - A fully charged battery will give you over 17 hours of use.

  - 14 hrs are required for a full battery charge.

- Take the filter to a Ludlum 3030 or iSOLO swipe counter.

- Follow the Instructions on FRM-0108F to fill in the remaining information.

- The remaining rows will be calculated on the electronic spreadsheet.

- Print a Hard copy.  Print and sign your name in RCT Name field.

- Turn in the completed spreadsheet to the Health & Safety Manager or designee to review the DAC levels.